Lab 6

Click here to Register Attendance

| Name |  |
| --- | --- |
| Date |  |
| Student No |  |
| Student Email |  |

### REST API Calls from a browser

- Connect your Yun to Wifi - and ping & note the IP address

- In a new browser window, type the IP address of your YUN & login to setup page

- Click the Configure button & at the bottom of the config page set REST API to Open

- From the Arduino IDE, run the Bridge sketch from File -> Examples -> Bridge

- Open a browser window and type the following to turn on LED 13: http://192.168.1.14/arduino/digital/13/1 
(replace the IP address with your Yun’s IP address)

- Note whether LED 13 lights up on your Yun and the message in your browser confirming

- Take a picture of your Yun & browser using your phone and insert here

- Now type the  the following to turn off LED 13: http://192.168.1.14/arduino/digital/13/1 
(replace the IP address with your Yun’s IP address)

- Once you have confirmed the URLs are working to turn LED 13 on and off, try saving this HMTL file to your PC/Laptop: https://github.com/marloft/ITSligoIoT/blob/main/LEDON.html

- Edit the hyperlinks in the HTML file to use your Yun’s IP address, save changes and open the HTML file in your browser

- Click the links to see if they turn LED 13 on and off

- Take a picture of your Yun & browser using your phone and insert here

### Blynk App

- Download the Blynk App to your phone

- Create a new Blynk Account in the App

- In your Arduino IDE, install the Blynk library by going to Sketch -> Include Library -> Manage Libraries, and searching for Blynk

- Open the Blynk App on your phone and start a new Project and select Arduino Yun as your hardware - you will be emailed an authentication code

- From File -> Examples -> Blynk -> Boards Wifi -> choose the Arduino Yun sketch

- Edit to include your authentication code and upload the sketch to your Yun

- Add a button control to your new Blynk Project

- For the button control, select output for D13 - Off value = 0, On value = 1

- Set Mode to Switch

- Press the Play Button at the top-right of the Blynk app to launch your app

- Test the button to see if it turns on/off your LED 13

- Explore other Blynk components and add one more to your app

- Add screenshots of your Blynk app and your Arduino responses here: